# Megaport Services Feature Catalog

This document outlines the key features and specifications of Megaport services including Port, Virtual Cross Connect (VXC), Cloud On-Ramp, and Megaport Cloud Router (MCR). Each section includes a feature description and a detailed table of specifications and expected values.

# Port

A Megaport Port is a physical Ethernet handoff at a Megaport-enabled data center that connects your network to the Megaport SDN.

| Specification | Description | Expected Values |
| --- | --- | --- |
| Port Speed | Physical interface capacity | 1 Gbps, 10 Gbps, 100 Gbps |
| Interface Optics | Supported SFP types | 1000BASE-LX, 10GBASE-LR, 100GBASE-LR4 |
| VLAN Trunking | Number of VXCs per Port via 802.1Q tagging | Up to 100 VXCs |
| Port Diversity | Redundant delivery | A-zone / B-zone, optional LAG (up to 8 links) |
| MTU Size | Maximum frame size | Up to 9100 bytes |
| Location Type | Data center where port is provisioned | Megaport-enabled global PoPs |

# Virtual Cross Connect (VXC)

A Virtual Cross Connect (VXC) is a private Layer 2 Ethernet circuit provisioned over the Megaport fabric between services.

| Specification | Description | Expected Values |
| --- | --- | --- |
| Connection Type | Endpoints between which VXC can be established | Port-to-Port, Port-to-MCR, Port-to-Cloud |
| Bandwidth Range | Adjustable connection capacity | 1 Mbps – 100 Gbps (in 1 Mbps steps) |
| Minimum Term | Duration of service commitment | Month-to-month, 12, 24, or 36 months |
| VLAN Tagging | Tagging mode for traffic segregation | Single tag (802.1Q), Q-in-Q (double tagging) |
| Setup Time | Time to provision the VXC once ordered | ~60 seconds (automated) |
| Encryption | Optional MACsec support (select metros only) | Enabled / Not Enabled |

# Cloud On-Ramp

Cloud VXCs enable direct, high-performance links from your network to major cloud providers using dedicated on-ramps.

| Specification | Description | Expected Values |
| --- | --- | --- |
| Supported Clouds | CSPs integrated into Megaport fabric | AWS, Azure, Google Cloud, Oracle, IBM |
| Connection Models | Cloud-specific VXC provisioning methods | Hosted VIF, Hosted Connection, ExpressRoute |
| Bandwidth Tiers | Connection capacity per cloud | 50 Mbps – 10 Gbps (varies by CSP) |
| Redundancy Support | Ability to create dual connections for HA | Supported (per CSP guidance) |
| Service Type | Cloud VPC/VNet/VLAN mapped via virtual interface | Private/Transit/Public interfaces |
| Regional Availability | Cloud on-ramp location coverage | Based on CSP presence & Megaport PoPs |

# Megaport Cloud Router (MCR)

Megaport Cloud Router (MCR) is a virtual Layer 3 router that simplifies multicloud and hybrid routing without hardware.

| Specification | Description | Expected Values |
| --- | --- | --- |
| MCR Speed Tiers | Allocated throughput for MCR | 1G, 2.5G, 5G, 10G, 25G, 50G, 100G |
| VXCs Supported | Number of virtual connections supported per MCR | Up to 100 VXCs |
| Routing Protocol | Protocol for dynamic exchange of routes | BGP (MD5, ASN, MED, communities) |
| Static Route Support | Optional static routing capability | Up to 100 routes |
| Subinterfaces | VLAN-tagged Q-in-Q interface support | Up to 5 per VXC |
| Use Cases | Cloud-to-cloud, hybrid cloud, cross-region routing | Any-to-any Layer 3 communication |
| Term Commitment | Duration of service | Month-to-month, 12, 24, or 36 months |

### 3 Cloud On-Ramp VXCs (key quick-order profiles)

| Cloud | Megaport connection model | Bandwidth tiers | Notes |
| --- | --- | --- | --- |
| AWS | Hosted VIF | 1 Mbps → 5 Gbps (1 Mbps steps) | Multiple VIFs per Port; auto-scales under Port cap. |
|  | Hosted Connection | 50 Mbps → 10 Gbps* | For private/public VIF; single logical link. |
| Azure | ExpressRoute | 50 Mbps → 10 Gbps* | Same VXC workflow; dual circuits supported. |
| Google Cloud | Partner Interconnect | 50 Mbps → 10 Gbps* | Choose VLAN attachment type (standard or HA). |
| Oracle, IBM, OCI, others | FastConnect / Direct Link etc. | 50 Mbps → 10 Gbps* | Same pay-as-you-go or termed billing. |

*Higher tiers (up to 50 G or 100 G) may be visible in limited regions; check portal for site-specific maxima.